	
	Handtevy
SKILL SHEET 
HP-CPR POSITION #3
	[image: image1.jpg]¥
o)






 NAME:_______________________________________________________________

Candidate shall perform the tasks and responsibilities of position #3 in an Adult/ Pediatric cardiac arrest scenario as outlined in the Handtevy HP-CPR schematic for cardiac arrest.
	
	Task Steps: 
	
	

	
	
	
	
	
	

	1.
	Manually open the airway. (suction airway if situation warrants)


	
	
	
	

	2.
	Deliver 2 big ventilations via BVM with an EtCO2 filter line to confirm the patient does not have a FBAO. EtCO2 filter line must be handed to

position #2. (Do not delay the 2 ventilations for EtCO2 filter line connection)


	
	
	
	

	3.
	Insert appropriate size i-gel by following the below sequence:
· Lubricate i-gel

· Size and pre-load gastric tube
· Insert i-gel
· Advance gastric tube

	
	
	
	

	4.
	Confirm proper i-gel placement by performing the following:

· Negative epigastric sounds then positive and equal lung sounds with ventilation

· Continuously monitor EtCO2 (provide verification with trend summary from monitor)

Assess for air leakage and reposition as necessary
	
	
	
	

	5.
	Secure in place with the equipment provided maintaining slight inward pressure. 


	
	
	
	

	6.
	Determine primary or secondary cardiac arrest.


	
	
	
	

	7. 
	If primary arrest, apply oxygen to the i-gel’s passive oxygenation port at 8 LPM and attach EtCO2 filter line to the i-gel’s 15mm connector.

	
	
	
	


	
	Prevent or prohibit any unsafe acts.
	Revised 01/19



